- International Europe Class Union

- Recommendations for Measurement at Major Events

draft 19. 5. 2009

- Please see also the “Measurement Instructions for Major Events”

- Station 1, Collect papers, entry and final control

The sailor arrives with the properly filled out “Measurement Entry Form”.

We check and collect this paper together with the Certificate and the Measurement Forms.

When the sailor believes that all is done and found to be correct,

he shall return the “Measurement Entry Form”.

The station will control the form and if found to be ok the Event Chief Measurer will sign them and inform the Race Committee.

You need:

at the beginning and each morning 3 persons, later on 2 persons,

these persons should be efficient in bureaucratic paperwork and the English language;

a desk of 2 meters length and 6 chairs;

blankets of the Measurement Entry Form;

folders to collect the papers for each competitor and boxes to store the folders;

blanket forms to help competitors in case of RRS 78.2.

- Station 2, Hull

The main work is weighing and the control of figures concerning security and figures related to attachments and rig.

You need:

at least 2 persons, better are 3 persons;

a space of 5 x 7 meters, wind protected:

a little desk and 3 chairs;

a balance with a range of appr. 90 kg, accuracy if possible +- 5 grams,

but we can live with +- 25 grams,

the balance should be certified and/or we need calibration weights;

plated lead and a scissor for cutting corrector weights (~ 5 kg);

a installation for checking the length and a water tank to check the floatability of the painter,

a possibility to mark the checked painter permanently.

- Station 3, Foils

- Checking centreboard, rudder and rudderstock.

You need:

2 persons;

a space of 4 x 4 meters;

a desk of 3 x 0.6 meters and 2 chairs:

a desk of 2 x 0.6 meters;

templates for the profiles of centreboard and rudder;

Installations to check the thickness of the blades;

a balance with a range of min 2 kg, accuracy within +- 1 gram;

a steel ruler of 150 mm;

a caliper.

- Station 4, Rig

- Checking mast and boom, at bigger events you need this station twice.

You need for each station:

a space of 8 x 4 meters;

3 persons;

a stiff and rigid desk of 6 x 0.9 meters with fittings as shown in the drawing, 3 chairs;

a balance with a range of min 6 kg, accuracy +- 5 grams;

plated lead and a scissor to cut corrector weights;

fast glue and tape to fix the afforded correctors to the spars;

Colour spray in white, yellow and black, tape to adjust the limit marks.

- Station 5, Sails

- Checking sails, at bigger events you need this station twice.

You need for each station:

a space of 7 x 5 meters;

3 persons;

a stiff and rigid melamin coated desk ~3.3 x 5 meters, cut like shown in the drawing, 3 chairs;

spare sail numbers and country figures;

2 steel rulers of 300 mm;

normally we do not stamp the sails, but when the committee wants to do so,

you need stamps, inkpads and waterproof ink.

- Station 6, Sample measurement

It is the intention of the Class to check at those big events the newest hulls of each brand

with the templates and swing test, similar as provided for prototype measurement.

This as a control of the market and the measurement standards in the different countries.

Errors found at this station normally do not influence the results of the Championship Measurement.

You need:

a space of 5 x 7 meters, wind protected;

3 qualified persons;

1 small desk and 3 chairs.

- Due to a deficit in manpower or insufficient preparation this station is often deleted.

- General remarks

Depending on the number of controlled items and the quality of the preparation and team of helpers you need 4 - 6 minutes for each piece of equipment.

Allowed are 1 hull, centreboard, rudder, rudderstock, mast, boom and 2 sails..

That means, 2 measurement days x 8 working hours a day x 60 minutes each hour divided through 6 minutes for each piece, each station may cover 160 pieces in two days.

If you have more, than you need more stations.

In practise we never had more than 2 stations.

The arrangement of the stations should be done in a way, that there is enough free space around so that boats and equipment may wait or go to another station as it suits to the go on of the work.

There should be 1 person responsible for entering the competitors and their equipment in the measurement area and the free flow inside.

Additionally you need a complete set of tools, screws and rivets to do all works for repairing or adjusting the gear, thinner, acetone, fast and slow glue, tape, paper towels and so on.

- Needed Manpower

The Event Chief Measurer, who is responsible for preparing and organizing, will work together with the Race Committee, the Jury and the Class Chief Measurer.

Both are not attached to a special station, but will help and enter whenever a problem will arise.

| Needed manpower for the team | Measurers or experienced helpers |  | Helpers |  |
| --- | --- | --- | --- | --- |
|  | first station | second station | first station | second station |
| Station 1, Papers | 2 |  | 1 |  |
| Station 2, Hull | 1 |  | 2 |  |
| Station 3, Foils | 2 |  |  |  |
| Station 4, Rig | 1 |  | 2 |  |
| Station 5, Sail | 2 | 1 | 2 | 2 |
| Station 6, Sample measurement | 2 |  | 1 |  |
| Entry control and inside free flow | 1 |  | 1 |  |
| Sums | 11 | 1 | 9 | 2 |
|  | 12 |  | 11 |  |
| All together | 23 |  |  |  |